Movements:

For all movements you first Disengage the safety and then press the desired move button:

StereoPylot SR

- Re-zero - performs a re-zero of the all 3 axis

- Goto pre-set working posit. – moves the axis to pre-defined position

- Full retract – moves all 3 axis away from working area

- ABS Zero – moves all 3 axis to towards the limit switch to ABS zero

- Rel.Zero (DV ABSzero) – moves to relative zero with DV moving all the way up to ABS zero

- Rel.Zero (DV up 5mm) – moves to relative zero with DV moving up 5mm

- Bregma – moves to relative zero position – DV is moved up 1.5mm from relative zero

- Lambda – moves 4mm posterior for mouse OR 9mm posterior for rat – DV is moved up 1.5mm from relative zero

- Retract AP – lifts DV up 10mm and moves AP anterior to make room

- Return AP – as long as position does not move once retracted will return AP to previous location with DV up 2mm

- Retract DV – lifts DV up to ABSzero

- Return DV – as long as position does not move once retracted will return DV to 1.5mm up from previous location

StereoPylot

- Re-zero - performs a re-zero of the all 3 axis

- Goto pre-set working posit. – moves the axis to pre-defined position

- Full retract – moves all 3 axis away from working area

- Lambda – moves 4mm posterior for mouse OR 9mm posterior for rat – DV is moved up 2mm from relative zero

- Bregma – moves to relative zero position – DV is moved up 2mm from relative zero

Species Select

StereoPylot SR

- Move the toggle switch to select mouse/null/rat

  - null will move DV up 1.5mm if Lambda movement is started

StereoPylot

- Disengage safety and press Species Select to switch between Mouse and Rat

Switching Offsets

For best results move back to relative zero (Bregma) before changing

StereoPylot SR

- Move the toggle switch to the desired offset

- Disengage safety

- Press Pre-set position button

StereoPylot

- Disengage safety

- Select the offset

Speed

Use the toggle to select the speed, no other buttons required.

Automated Movements:

StereoPylot SR

3 methods of automated movement

- MakeItSo - Move to X,Y, and Z via manual entry or from list.

- DV insert – Move to DV depth from entry from list

- Withdrawl – Withdraw based on parameters from list

- Disengage safety

- Select desired movement (MakeItSo, DV insert, Withdraw) within 4 seconds

- Press Engage within 4 seconds of selecting the movement selection

- To STOP automated movement of DV insert or Withdrawl

  - Press Disable Stepper button OR Safety Disengage

StereoPylot

1 methods of automated movement

- MakeIsSo - Move to X,Y, and Z via manual entry or from list.

- Using the keypad

- Select (or enter) the desired coordinates

- check Make It So checkbox

- press Engage button